Research Paper Report

陳以薰 Chynna

Paper 1: review paper

Thermal Modeling and Thermal-Aware Energy Saving Methods for Cloud Data Centers

- Background and Problem

Data centers consume a large amount of energy, where cooling systems account for approximately 37% of total energy consumption. High temperature can degrade performance and reduce hardware lifespan.

- Core Problems:
a. Complex thermal distribution (nonlinear, time-varying)
b. Coupling between IT workload and cooling system
c. High-dimensional optimization problem

Formula: Etotal = EIT + Ecooling

- Thermal Management Framework

Sensors collect temperature, airflow, and power data in IT rooms, which are sent to the IoT-based Data Platform. The thermal model predicts temperature distribution, and the cloud manager adjusts IT scheduling and cooling parameters accordingly.

3. Thermal Modeling Methods

- White-box model: based on physical laws, high accuracy but complex.

- Black-box model: data-driven (ML), fast but depends on data.

- Gray-box model: combines physical and data-driven methods, balancing accuracy and efficiency.

4. Energy Saving Methods

- IT optimization: VM scheduling, workload allocation, DVFS.

- Cooling optimization: fan control, chilled water control.

- Joint optimization: optimize IT and cooling systems simultaneously for global energy reduction.

- Static Joint Optimization (SJO): heuristic、genetic algorithm、optimization solver : 決定最佳參數組合

- Dynamic Joint Optimization (DJO): Reinforcement Learning (RL)、Deep Reinforcement Learning (DRL): Markov Decision Process (MDP)

6. Challenges and Future Work

- Challenges:

- Accurate thermal modeling

- System coupling complexity

- Real-time control difficulty

- Future directions:

- Gray-box modeling

- Multi-agent control

- Liquid cooling technologies